AT1 – Engineering Design Project

ENG112 | Sustainable and Life-centred Design

RAT Trap Boat

NAMES: NORa bolme fjerstad (270333)

DATE

Progress:

Part 1 – Introduction & Problem Definition à Done

Part 2 – OFFERS Problem Analysis à Almost done? But make it more presentable

Part 3 – Concept Generation (Morphology) à Almost done? But make it more presentable

Part 4 – Concept Evaluation & Selection à Done

Part 5 – Engineering Analysis of Selected Concept à Not started

Part 6 – Final Concept Description (CAD) à Started

Part 7 – Conclusion à Not started

Part 8 – References

Area of responsibilities:

Part 1 – Introduction & Problem Definition à NORA

Part 2 – OFFERS Problem Analysis à DANIEL

Part 3 – Concept Generation (Morphology) à YUAN

Part 4 – Concept Evaluation & Selection à NORA

Part 5 – Engineering Analysis of Selected Concept à YUAN AND CHAMOD

Part 6 – Final Concept Description (CAD) à HESH

Part 7 – Conclusion à DANIEL AND CHAMOD

Part 8 – References à EVERYONE

Part 1 – Introduction & Problem Definition

This project is based on a rat trap boat (RTB) challenge, where student groups handle the design, build and race a boat which get power from a single rat trap. The original RTB challenge comes from the Australian Maritime College and is used here in this report to work on concept generation, evaluation, analysis and lastly construction. There are multiple requirements for the boat when it comes to manufacturing and design. These requirements will be checked when the boat is racing.

However, before it's possible to test and actually make the boat based on a design, it's important to define the engineering problem. This makes it possible to not limit any concept to early in the design process. Based on the project brief the engineering problem can be defined as finding a way to convert mechanical energy into movement that makes a small boat move forward. With a goal of moving as fast as possible at a known distance. This need to happen simultaneous while the boat is stable, floats, and has the allowed dimensions.

The dimensions given in the project brief include a single rat trap as energy source. The maximum length of the boat is 1000mm, it needs to be wider than 500mm and taller than 600mm above water level, as well as 200 mm below it. Lastly the weight needs to be at least 1000g. Most common materials can be used, however it's not allowed with exotic materials and no CNC machining or laser.  3D printing is allowed but can only be up to 200g per group.

Based on these rules and dimensions, the first goal of the design process it to make sure that the rat trap boat floats and has all the structurally matters in place. This is the most important aspect, because having it in place means we can proceed with the rest of the process. After this, efficiency become the focus, with the goal that the boat needs to go as fast as possible on an 8m course, while being within the project requirements and limits.

Part 2 – OFFERS Problem Analysis

Objective

Build a boat that can float for 5 minutes, propel itself under the power of a rat trap, and travel in a straight line for 8 meters without any components being CNC machined, laser cut or made out of exotic materials.

Functions:

Move under power of the rat trap

Travel in a straight line for 8 meters

Float, and move in a straight line as fast as possible

Use stored energy to move the boat

Store mechanical energy

Factors:

Men: Mechanical Engineering knowledge
Money: budget

Machines: No cnc machining or laser cutting.

Methods: 3D printing, cutting, carving, gluing, nailing

Materials: Any commonly available construction materials, 2 rat traps, thin coat of water-proof paint

Minutes: 5 weeks or 5min?

Effects:

Safety: adhere to the general WHS requirements

Environmental: not pollute the wave tank

Reliability:

Requirements:

Materials: no exotic materials, nothing that will damage or pollute the wave tank

Propulsion: exclusively given from the rat trap

Construction: No CNC machining or laser cutting, PLA 3d printing permitted under 200g of filament.

Specifications:

Maximum size: 1000mm length, 500mm width, 600mm height above water line and 200mm below water line, minimum mass of 1000g

Part 3 – Concept Generation (Morphology)

A morphological analysis was used to generate possible concepts for the rat trap boat. The design was divided into key sub-systems so that different solutions could be considered for each area before being combined into complete concepts

Key Functions and Sub-systems

The key-sub systems identified for concept generation were the hull form, propulsion method, transmission mechanism, rat trap mounting position, steering method, hull material and centre of mass. These were selected because they directly affect the flotation, propulsion, stability, manufacturability and overall performance of the boat.

Morphological Analysis

Several possible solutions were considered for each sub-system, as shown in Table 1. Different combinations of these options were then used to generate distinct boat concepts.

Table 1 - Morphological Analysis (Concept Generation)

| Function/Sub-system | Option A | Option B | Option C | Option D |
| --- | --- | --- | --- | --- |
| Hull form | Trimaran | Catamaran | Flat bottom hull | Round hull |
| Propulsion method | Paddlewheel | Propeller | Sail |  |
| Transmission mechanism | Gears | String/belts | None |  |
| Rattrap mounting | Stern | Middle of deck | Bow |  |
| Steering | Small Styrofoam balls protruded out at the bow of boat at 45-degree angle | None | Rudder |  |
| Hull Material | Styrofoam | Wood | Plastic |  |
| Centre of mass | Low | Middle | High |  |

Concept 1: Catamaran with Paddlewheel (B, A, B, B, A, A, B)

Concept 1 consists of a catamaran hull made from polystyrene foam, with a paddlewheel used for propulsion. Energy from the rat trap is transferred to the paddlewheel using a string or belt transmission, with the rat trap positioned near the middle of the deck. Angled foam guides are positioned near the bow to assist the boat in maintaining a straight direction.

Concept 2: Trimaran with Paddlewheel (A, A, A, B, B, A, A)

Concept 2 uses a trimaran hull made from polystyrene foam, with a paddlewheel providing propulsion. The rat trap is mounted near the middle of the deck, and its stored energy is transferred to the paddlewheel through a gear transmission. A low centre of mass is used to improve the stability of the boat.

The three-hull arrangement provides multiple buoyant sections and a wide platform for the main components. However, additional construction is required compared with the catamaran because three hull sections and a gear transmission must be produced and correctly aligned.

Concept 3: Round Hull with Sail (D, C, C, C, B, B, C)

Concept 3 consists of a round hull made from wood, with a sail used as the propulsion method. The rat trap is positioned towards the bow, with no additional steering system included. A high centre of mass is used in this concept.

The round hull provides a different hull arrangement compared with the multi-hull concepts. However, the high centre of mass may reduce stability, while the sail provides less reliable propulsion must be provided exclusively by the rat trap, making it less suitable for further development.

Concept 4: Catamaran with Propeller (B, B, A, A, C, C, A)

Concept 4 consists of a catamaran hull made from plastic, with a propeller used for propulsion. Energy from the rat trap is transferred to the propeller through a gear mechanism, with the rat trap positioned towards the stern. A rudder is included to provide directional control, while a low centre of mass is used to improve stability.

The catamaran hull provides a stable platform for the propulsion system, while the propeller and rudder allow propulsion and directional control. However, the gears, propeller shaft and rudder increase the number of components required and make manufacturing and alignment more difficult compared with the simpler paddlewheel concepts.

Part 4 – Concept Evaluation & Selection

The concepts are evaluated in this part by using the criteria in the table below. The criteria that were used was: efficiency, float/weight, stability, manufacturability, environmental impact and rule following. All the weightings were based on requirements and goals given in the project brief and are taken from the OFFERS analysis, presented in part 2.

The boats efficiency was given a weighting of 30%, because the main goal of the boat is that it travels 8m as fast as possible (from the project brief). Float/weight had the second highest weighting at 25%, since the rat trap boat needs have the minimum mass requirement and float for at least 5 minutes.

Stability and manufacturability both were given a 15% weighting, because if the boat is hard to build or is unstable when placed in the water, it may not work well. Rule following was weighted at 10%, so the boat design is compliant with the competition rules. Lastly is the environmental impact that was weighted at 0.05%, because it felt less important compared to the other criterions, that would result in a working rat trap boat.

Table 1, evaluation criteria and weighting

| Criteria | Weighting |
| --- | --- |
| Efficiency | 0.3 (30%) |
| Floats / Weight | 0.25 (25%) |
| Stability | 0.15 (15%) |
| Manufacturability | 0.15 (15%) |
| Rule following | 0.1 (10%) |
| Environmental impact | 0.05 (5%) |
| Sum | 1 (100%) |

Composite Criterion Evaluation

Each concept from the concept generation (part 3) was scored out of 10 based on the six criteria presented above. Then that score was multiplied by the weighting, which gave the weighted value. All the four concept and their results are listed in the tables below.

| Criterion | Weighting | Concept 1 |  | Concept 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Score | Weighted Value | Score | Weighted Value |
| Efficiency | 0.3 | 8 | 2.4 | 7 | 2.10 |
| Floats / Weight | 0.25 | 8 | 2.0 | 8 | 2.0 |
| Stability | 0.15 | 8 | 1.20 | 9 | 1.35 |
| Manufacturability | 0.15 | 9 | 1.35 | 6 | 0.90 |
| Environmental impact | 0.05 | 9 | 0.45 | 9 | 0.45 |
| Rule following | 0.1 | 9 | 0.90 | 9 | 0.90 |
| Overall score | 1 |  | 8.30 |  | 7.70 |

Table 2, all values from concept 1 and 2

| Criterion | Weighting | Concept 3 |  | Concept 4 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Score | Weighted Value | Score | Weighted Value |
| Efficiency | 0.3 | 4 | 1.20 | 9 | 2.70 |
| Floats / Weight | 0.25 | 5 | 1.25 | 7 | 1.75 |
| Stability | 0.15 | 5 | 0.75 | 8 | 1.20 |
| Manufacturability | 0.15 | 7 | 1.05 | 6 | 0.90 |
| Environmental impact | 0.05 | 7 | 0.35 | 8 | 0.40 |
| Rule following | 0.1 | 5 | 0.50 | 9 | 0.90 |
| Overall score | 1 |  | 5.10 |  | 7.85 |

Table 3, all values from concept 3 and 4

Reflection and Justification

Based on the composite criterion evaluation and the results in the tables above, Concept 1 achieved the highest weighted score of 8.30. Concept 1 uses a catamaran hull with a paddlewheel propulsion system and a string/belt transmission. This scored highly because the catamaran provides good stability and flotation while remaining relatively simple to manufacture, which gives a high score one the stability and manufacturing criteria.

Concept 4 was the second-highest scoring design. Its propeller may provide better propulsion efficiency than a paddlewheel, but the gears, propeller shaft and rudder would increase the complexity of manufacturing and require more accurate design. This also reflects why this concept got a low manufacturing score compared to concept 1. Concept 2 had a similar score as concept 4, but had a lower score on efficiency, float/weight and stability that reflect to a lover overall score.

Concept 3 received the lowest score. This is mainly because of its round hull, sail propulsion and high centre of mass. These factors would provide lower stability and less reliable force from the rat trap. However, the criteria that lower the overall score most is the rule following. The sail propulsion does not follow the competitions rule that a rat trap must be used to create propulsion.

Based on the composite criterion evaluation, concept 1 was chosen for further improvement and CAD modelling.

Part 5 – Engineering Analysis of Selected Concept (a) Flotation

• Estimate total mass and displaced volume.

• Determine draft and waterline. (b) Dynamics

• State available energy and efficiency assumption.

• Estimate speed and 8 m run time. (c) Lifecycle & Life-Centred Design

• Identify materials and end-of-life pathways.

• Identify design decisions affecting disassembly and recovery.

• Justify material choices using lifecycle reasoning.

Part 5(c) – Lifecycle and Life-Centred Design

Lifecycle considerations were included in the selected design to reduce material waste and allow components to be recovered after the boat has been tested. The main materials expected to be used in Concept 1 included expanded polystyrene (EPS) for the hulls, wood, and steel from the rat trap, string or belt material for the transmission, and potentially small amounts of plastic or wood for the paddlewheel and supporting components.

Table ? Materials and proposed end-of-life pathways

| Component | Material | End-of-life pathway |
| --- | --- | --- |
| Hulls | EPS foam | Reuse or specialised EPS recycling |
| Rat trap | Wood/steel | Reuse or separate materials for recovery |
| Paddlewheel | Wood/plastic | Reuse or recycle where possible |
| Axle/supports | Wood/steel | Reuse or recycle |
| Transmission | String/belt | Reuse where possible |
| 3d-printed parts | PLA | Reuse or separate for appropriate recovery |

EPS was selected for the hulls because its low mass allows a large buoyant volume to be provided without significantly increasing the overall mass of the boat. However, EPS has limitations at its end-of-life because it is not generally accepted through Australian kerbside recycling systems. Clean EPS can instead be collected and reprocessed where suitable recycling facilities are available.

Disassembly was also considered in the design. Permanent adhesive connections between different materials should be minimised where practical. Removable or mechanical connections can instead be used so that the rat trap, propulsion system and hull sections can be separated after testing. Designing products so that components can be disassembled improves opportunities for reuse, recycling and material recovery.

The material choices were considered across both the use and end-of-life stages of the boat. EPS was selected because it provides suitable flotation while keeping the overall mass low, although its limited recycling options mean that reuse of the hulls should be prioritised. Reusable materials such as wood and steel were preferred for structural and mechanical components because these parts can be recovered after testing. The relatively simple paddlewheel and string transmission also requires fewer specialised components than the geared propeller concept, reducing material use and making disassembly easier. Overall, materials were selected to provide the required performance while allowing components to be reused or recovered where practical.

Part 6 – Final Concept Description (CAD)

• Include labelled CAD views: side, plan, isometric.

• Identify major components.

• Link design features to performance.

Part 7 – Conclusion

• Recommend final design concept.

• Comment on analysis limitations and inefficiencies.

• Suggest realistic improvements.

Part 8 – References

• List all cited sources using IEEE style.